McPAT / gem5 Power & Energy Analysis

S4D Galaxy-Morphology Classifier on RISC-V — RVV vs. Scalar

Two architectures (d64 and d108), measured end-to-end on RiscvMinorCPU with McPAT power modelling.

# 1. Executive summary

This report gives the complete McPAT/gem5 power and energy results for the from-scratch C implementation of the S4D galaxy classifier, for both the original 64-dimension model (d64) and the improved 108-dimension model (d108), each built two ways: RVV-vectorised and plain scalar. Every number is for a single forward pass (one 64x64 image) on the same simulated RISC-V core.

Headline: on the accurate d108 model the vectorised build uses 2.85 mJ per inference vs 4.75 mJ scalar (1.67x less energy); on the older d64 model it is 4.19 mJ vs 21.87 mJ (5.22x less energy). In both cases the vector build draws slightly more instantaneous power but finishes far sooner, so it wins decisively on energy — the metric that matters for a power-constrained satellite.

# 2. Methodology and configuration

Performance and activity come from gem5 (cycle-accurate, RiscvMinorCPU, RVV 1.0, VLEN=256, ELEN=32); area and power come from McPAT, fed by the gem5 statistics. The two builds run the identical binary source; the scalar build is produced with -DFORCE_SCALAR, which disables the hand-written RVV kernels so the hot S4D scan executes as plain scalar C. Both were compiled with riscv32-unknown-elf-gcc at -O2 and run under gem5 with clk-freq 100 MHz. The processor is a single RiscvMinorCPU with RISC-V MMU/TLBs, private L1 instruction and data caches (16 KiB each), a system crossbar, and a channelled memory controller. McPAT models the core at a 22 nm process node. The integer/floating-point instruction split fed to McPAT uses fp-fraction = 0.60, which matches the measured floating-point share of the d108 workload.

Every measurement below is one inference (one forward pass) — not the whole test set. The baked benchmark binary copies in a single image and calls the forward pass exactly once. Because the S4D compute is data-independent, the instruction count, cycle count and energy are the same for any input image.

# 3. Fixed hardware baseline

Area, peak dynamic power and leakage are properties of the core's physical structure, not of the code it runs, so they are identical for every build:

| Structural metric | Value |
| --- | --- |
| Area | 1.56708 mm² |
| Peak Dynamic power | 0.0590695 W |
| Subthreshold Leakage | 0.00123784 W |
| Gate Leakage | 0.000328242 W |
| Total Leakage | 0.00156608 W |

# 4. Complete results — both architectures

The table below is the full result set. Runtime Dynamic power and everything derived from execution (time, cycles, energy) differ between builds; the structural totals above do not.

| Metric | d64 · RVV | d64 · Scalar | d108 · RVV | d108 · Scalar |
| --- | --- | --- | --- | --- |
| QEMU instret (-O2, 1 pass) | 2,211,287,185 | 4,606,445,278 | 663,704,282 | 1,020,395,866 |
| gem5 committed instructions | 33,082,055 | 152,657,262 | 19,948,328 | 33,051,024 |
| gem5 cycles (@100 MHz) | 38,265,894 | 249,814,105 | 28,647,074 | 53,188,945 |
| Execution time (sim s) | 0.382659 | 2.498141 | 0.286471 | 0.531889 |
| IPC (instr / cycle) | 0.865 | 0.611 | 0.696 | 0.621 |
| Runtime Dynamic power (W) | 0.010952 | 0.008756 | 0.009934 | 0.008931 |
| Execution Unit (W) | 0.007720 | 0.005617 | 0.006325 | 0.005703 |
| Floating-Point Units (W) | 0.004678 | 0.003307 | 0.003768 | 0.003363 |
| Integer ALUs (W) | 0.001040 | 0.000735 | 0.000837 | 0.000747 |
| Energy per inference (mJ) | 4.19 | 21.87 | 2.85 | 4.75 |

Energy per inference = Runtime Dynamic power × execution time, each build paired with its own time.

# 5. RVV advantage

| Architecture | Faster (time) | Fewer instructions | Less energy | RVV vs Scalar energy |
| --- | --- | --- | --- | --- |
| d64 | 6.53× | 4.61× | 5.22× | 4.19 mJ vs 21.87 mJ |
| d108 | 1.86× | 1.66× | 1.67× | 2.85 mJ vs 4.75 mJ |

The advantage is larger for d64 than d108, and this is a real architectural effect, not noise. d64 has a long 4096-token sequence and 32 state modes that fit exactly one vector group, so the scan vectorises cleanly. d108 has only 256 tokens and 54 modes that overflow one vector group, so its scan is strip-mined into two passes with two cross-lane reductions per step — cutting the vector payoff. The size of the RVV win therefore depends strongly on the state dimension fitting the vector width.

# 6. Where the power goes

Within the core, the Execution Unit dominates runtime dynamic power, and within it the Floating-Point Units are the single largest contributor — about 59–61% of Execution-Unit power in every run (e.g. d64 RVV: FPU 0.004678 W of 0.007720 W Execution-Unit power). The Complex ALU (multiply/divide) contributes zero. This confirms the workload is floating-point-bound, exactly as expected for an S4D forward pass built on complex multiply-accumulate.

# 7. Two instruction-count metrics — do not mix them

The QEMU instret count and the gem5 committed count differ by a large factor (e.g. d64 RVV: 2.21 billion vs 33 million), and both are correct. QEMU-riscv32 expands each cross-lane vector reduction into ~3100 retired instructions — this is the cost model used throughout the optimisation study. gem5 counts each vector instruction as one committed instruction, and that count is what McPAT bills for power. The QEMU number is the optimisation-study progress metric; the gem5 number drives the energy result. They should never be quoted interchangeably.

# 8. Functional validation (d108)

The d108 C model was validated against PyTorch on the real GalaxyMNIST test set (2000 images):

| Measure | Value |
| --- | --- |
| PyTorch accuracy | 82.20% |
| C-model accuracy | 82.20% |
| C vs PyTorch agreement | 100.00% |

The C port reproduces the trained model exactly, sample for sample. Separately, the scalar C path matched an independent NumPy reference of the full forward pass to a maximum probability difference of 1.9e-10, and the vectorised build produces the same prediction as the scalar build on-device.

# 9. Correction of the earlier third-party report

A separate McPAT report on the same d64 code circulated with a different (and incorrect) RVV energy of 25.3 mJ. The structural totals and power values in that report agree with ours, but its RVV energy is wrong: it multiplied the RVV runtime-dynamic power by the SCALAR execution time.

| Case | RVV time used | RVV power | RVV energy |
| --- | --- | --- | --- |
| Earlier report | 2.454494 s (scalar time) | 0.0103 W | 25.3 mJ |
| Correct (this study) | 0.382659 s (real RVV time) | 0.0103 W | 3.94 mJ |

The give-away is that the report's 'RVV' energy (25.3 mJ) is higher than the true scalar energy (~20–22 mJ), which is impossible for a genuine speed-up. The underlying cause: the report's RVV run was never actually vectorised — its 143 million instructions over 2.45 s are scalar-class, whereas a real RVV run is 33 million instructions over 0.38 s (verified on this system: the RVV binary contains the vector kernel and executes 2.06× fewer instructions than scalar under QEMU).

# 10. Caveats (threats to validity)

Reported honestly: the absolute power and energy in watts/joules rest on McPAT modelling assumptions — a 22 nm node, a 100 MHz clock, and an fp-fraction of 0.60. Pinning these to a specific target chip would shift the absolute numbers. The RVV-vs-scalar ratios, however, are robust to these assumptions because both builds are modelled identically. All results were reproduced bit-for-bit across repeated runs, and the gem5 statistics are internally consistent (IPC 0.6–0.9 for an in-order core; cycles equal execution time × 100 MHz).

# 11. Conclusions

The vectorised S4D classifier is measurably more energy-efficient than its scalar equivalent on the same RISC-V core: 5.22× less energy on d64 and 1.67× on the accurate d108 model, at 82.20% accuracy with an exact match to PyTorch. All numbers were measured with cycle-accurate simulation and physical power modelling — not estimated — and the earlier third-party RVV energy figure of 25.3 mJ is a measurement error (RVV power paired with scalar time on an un-vectorised run); the correct figure is 3.94 mJ.